20. MQTT

Interview Learning Guide with Complete .NET WPF Implementation

Goal of this document Understand MQTT from zero to production interview level: concept, internal working, when to use, when not to use, trade-offs, production issues, and a small complete implementation using a WPF dashboard and a sensor simulator.

# Learning Roadmap

Concept and mental model

MQTT architecture and internal flow

Topics, QoS, retained messages, and Last Will

Complete working implementation on Windows 11 without Docker

WPF dashboard subscriber and console sensor publisher

How to run both projects in Visual Studio

Production issues, trade-offs, and interview answers

# 1. Concept

MQTT stands for Message Queuing Telemetry Transport. It is a lightweight messaging protocol based on the publish-subscribe pattern. It is commonly used when devices or services need to send small messages reliably over low-bandwidth or unstable networks.

In MQTT, producers are called publishers, consumers are called subscribers, and messages are routed through an MQTT broker. Publishers do not directly know subscribers. They only publish messages to a topic.

Publisher/Sensor  --->  MQTT Broker  --->  Subscriber/WPF Dashboard

Example topic:

factory/line1/machine1/temperature

Simple interview definition MQTT is a lightweight publish-subscribe protocol used mostly in IoT, manufacturing, sensors, and telemetry systems. Devices publish messages to topics, and subscribers receive messages through a broker.

# 2. Why MQTT Instead of REST?

| Area | REST | MQTT |
| --- | --- | --- |
| Communication model | Request-response | Publish-subscribe |
| Connection | New or managed HTTP request per call | Persistent TCP connection |
| Payload size | Usually heavier headers | Small packet overhead |
| Best for | Business APIs, CRUD operations | Telemetry, sensors, device events |
| Offline handling | Application must design it | Protocol supports persistent sessions and retained messages |

# 3. Architecture for Our Demo

We will build a small factory temperature monitoring system. It will run locally on Windows 11 without Docker.

TemperatureSensorSimulator Console App

|

| publishes JSON every 2 seconds

v

Local Mosquitto MQTT Broker on localhost:1883

|

| forwards messages by topic

v

TemperatureDashboard WPF App

## Topics Used

factory/line1/machine1/temperature

factory/line1/machine1/status

factory/line1/machine1/alert

## Functionality Covered

Publish and subscribe

Topic hierarchy

QoS 1 delivery

Retained machine status

Last Will and Testament

Auto reconnect

UI update from MQTT callback using WPF Dispatcher

Simple alert detection in WPF

# 4. How MQTT Works Internally

| Step | Internal Meaning |
| --- | --- |
| CONNECT | Sensor and WPF client open TCP connection to broker. |
| SUBSCRIBE | WPF dashboard subscribes to temperature and status topics. |
| PUBLISH | Sensor publishes temperature JSON to the temperature topic. |
| TOPIC MATCHING | Broker checks which clients subscribed to that exact topic or wildcard pattern. |
| FORWARD | Broker forwards messages to matching subscribers. |
| ACK | With QoS 1, broker/client acknowledges message delivery. If acknowledgement is missing, message may be retried. |

# 5. Important MQTT Features

## 5.1 QoS - Quality of Service

| QoS | Meaning | Behavior | Use Case |
| --- | --- | --- | --- |
| 0 | At most once | No retry, fastest, message can be lost | Frequent telemetry where one missed reading is acceptable |
| 1 | At least once | Retry until ACK, duplicates possible | Alarms, machine events, important telemetry |
| 2 | Exactly once | Four-step handshake, slowest | Critical operations where duplicates are unacceptable |

## 5.2 Retained Message

A retained message is the latest value stored by the broker for a topic. New subscribers immediately receive it. In our demo, machine status uses retained messages so WPF can immediately know whether the machine is ONLINE or OFFLINE.

WithRetainFlag(true)

Topic: factory/line1/machine1/status

Value: ONLINE or OFFLINE

## 5.3 Last Will and Testament

Last Will and Testament, usually called LWT, is a message configured by the client at connection time. If the client disconnects unexpectedly, the broker automatically publishes the configured will message. In our demo, if the sensor app crashes, broker publishes OFFLINE.

WithWillTopic(statusTopic)

WithWillPayload("OFFLINE")

WithWillRetain(true)

# 6. Complete Implementation - Windows 11 Without Docker

## 6.1 Prerequisites

Windows 11

Visual Studio 2022

NET 8 SDK

Mosquitto MQTT broker installed locally

MQTTnet NuGet package

## 6.2 Install Mosquitto Broker on Windows

Download and install Mosquitto for Windows from the official Mosquitto download page. Usually it installs under C:\Program Files\mosquitto.

C:\Program Files\mosquitto

Create or update mosquitto.conf in that folder with this local development configuration:

listener 1883

allow_anonymous true

Start broker from Command Prompt as Administrator:

cd "C:\Program Files\mosquitto"

mosquitto.exe -c mosquitto.conf -v

Keep this command window open. Your broker is now running on localhost:1883.

## 6.3 Create Visual Studio Solution

mkdir MqttTemperatureDemo

cd MqttTemperatureDemo

dotnet new sln -n MqttTemperatureDemo

dotnet new console -n TemperatureSensorSimulator

dotnet new wpf -n TemperatureDashboard

dotnet sln add TemperatureSensorSimulator/TemperatureSensorSimulator.csproj

dotnet sln add TemperatureDashboard/TemperatureDashboard.csproj

dotnet add TemperatureSensorSimulator package MQTTnet --version 4.3.7.1207

dotnet add TemperatureDashboard package MQTTnet --version 4.3.7.1207

# 7. Project 1 - TemperatureSensorSimulator Console App

This project simulates a machine sensor. It connects to the MQTT broker, publishes ONLINE status as a retained message, registers Last Will as OFFLINE, and publishes temperature every 2 seconds.

## 7.1 TemperatureSensorSimulator.csproj

<Project Sdk="Microsoft.NET.Sdk">

<PropertyGroup>

<OutputType>Exe</OutputType>

<TargetFramework>net8.0</TargetFramework>

<ImplicitUsings>enable</ImplicitUsings>

<Nullable>enable</Nullable>

</PropertyGroup>

<ItemGroup>

<PackageReference Include="MQTTnet" Version="4.3.7.1207" />

</ItemGroup>

</Project>

## 7.2 Program.cs

using System.Text;

using System.Text.Json;

using MQTTnet;

using MQTTnet.Client;

using MQTTnet.Protocol;

Console.WriteLine("Starting Temperature Sensor Simulator...");

var factory = new MqttFactory();

var mqttClient = factory.CreateMqttClient();

var machineId = "machine1";

var lineId = "line1";

var temperatureTopic = $"factory/{lineId}/{machineId}/temperature";

var statusTopic = $"factory/{lineId}/{machineId}/status";

var options = new MqttClientOptionsBuilder()

.WithTcpServer("localhost", 1883)

.WithClientId($"sensor-{machineId}")

.WithCleanSession(false)

.WithWillTopic(statusTopic)

.WithWillPayload("OFFLINE")

.WithWillQualityOfServiceLevel(MqttQualityOfServiceLevel.AtLeastOnce)

.WithWillRetain(true)

.Build();

mqttClient.ConnectedAsync += async e =>

{

Console.WriteLine("Sensor connected to MQTT broker.");

var onlineMessage = new MqttApplicationMessageBuilder()

.WithTopic(statusTopic)

.WithPayload("ONLINE")

.WithQualityOfServiceLevel(MqttQualityOfServiceLevel.AtLeastOnce)

.WithRetainFlag(true)

.Build();

await mqttClient.PublishAsync(onlineMessage);

Console.WriteLine($"Published ONLINE status to topic: {statusTopic}");

};

mqttClient.DisconnectedAsync += async e =>

{

Console.WriteLine("Sensor disconnected from MQTT broker.");

await Task.Delay(TimeSpan.FromSeconds(5));

try

{

Console.WriteLine("Trying to reconnect sensor...");

await mqttClient.ConnectAsync(options);

}

catch

{

Console.WriteLine("Reconnect failed.");

}

};

await mqttClient.ConnectAsync(options);

var random = new Random();

while (true)

{

var temperature = Math.Round(65 + random.NextDouble() * 30, 2);

var payload = new TemperatureReading

{

MachineId = machineId,

LineId = lineId,

Temperature = temperature,

TimestampUtc = DateTime.UtcNow

};

var json = JsonSerializer.Serialize(payload);

var message = new MqttApplicationMessageBuilder()

.WithTopic(temperatureTopic)

.WithPayload(Encoding.UTF8.GetBytes(json))

.WithQualityOfServiceLevel(MqttQualityOfServiceLevel.AtLeastOnce)

.WithRetainFlag(false)

.Build();

await mqttClient.PublishAsync(message);

Console.WriteLine($"Published: {json}");

await Task.Delay(TimeSpan.FromSeconds(2));

}

public class TemperatureReading

{

public string MachineId { get; set; } = "";

public string LineId { get; set; } = "";

public double Temperature { get; set; }

public DateTime TimestampUtc { get; set; }

}

# 8. Project 2 - TemperatureDashboard WPF App

This project is the frontend. It connects to MQTT, subscribes to temperature and status topics, and updates the UI in real time.

## 8.1 TemperatureDashboard.csproj

<Project Sdk="Microsoft.NET.Sdk">

<PropertyGroup>

<OutputType>WinExe</OutputType>

<TargetFramework>net8.0-windows</TargetFramework>

<Nullable>enable</Nullable>

<UseWPF>true</UseWPF>

<ImplicitUsings>enable</ImplicitUsings>

</PropertyGroup>

<ItemGroup>

<PackageReference Include="MQTTnet" Version="4.3.7.1207" />

</ItemGroup>

</Project>

## 8.2 MainWindow.xaml

<Window x:Class="TemperatureDashboard.MainWindow"

xmlns="http://schemas.microsoft.com/winfx/2006/xaml/presentation"

xmlns:x="http://schemas.microsoft.com/winfx/2006/xaml"

Title="MQTT Temperature Dashboard"

Height="450"

Width="700"

WindowStartupLocation="CenterScreen">

<Grid Margin="20">

<Grid.RowDefinitions>

<RowDefinition Height="Auto"/>

<RowDefinition Height="Auto"/>

<RowDefinition Height="Auto"/>

<RowDefinition Height="*"/>

</Grid.RowDefinitions>

<TextBlock Text="Factory Temperature Dashboard"

FontSize="26"

FontWeight="Bold"

Margin="0,0,0,20"/>

<StackPanel Grid.Row="1" Orientation="Horizontal" Margin="0,0,0,20">

<Button x:Name="ConnectButton"

Content="Connect"

Width="120"

Height="35"

Margin="0,0,10,0"

Click="ConnectButton_Click"/>

<Button x:Name="DisconnectButton"

Content="Disconnect"

Width="120"

Height="35"

Click="DisconnectButton_Click"/>

</StackPanel>

<Border Grid.Row="2" BorderBrush="Gray" BorderThickness="1"

CornerRadius="8" Padding="20" Margin="0,0,0,20">

<StackPanel>

<TextBlock x:Name="ConnectionStatusText"

Text="Broker Status: Disconnected"

FontSize="16"

FontWeight="SemiBold"

Margin="0,0,0,10"/>

<TextBlock x:Name="MachineStatusText"

Text="Machine Status: Unknown"

FontSize="16"

Margin="0,0,0,10"/>

<TextBlock x:Name="TemperatureText"

Text="Temperature: --"

FontSize="40"

FontWeight="Bold"

Margin="0,0,0,10"/>

<TextBlock x:Name="AlertText"

Text="Alert: None"

FontSize="18"

FontWeight="Bold"

Foreground="Green"/>

</StackPanel>

</Border>

<ListBox x:Name="LogListBox" Grid.Row="3"/>

</Grid>

</Window>

## 8.3 MainWindow.xaml.cs

using System.Text;

using System.Text.Json;

using System.Windows;

using MQTTnet;

using MQTTnet.Client;

using MQTTnet.Protocol;

namespace TemperatureDashboard;

public partial class MainWindow : Window

{

private readonly IMqttClient _mqttClient;

private readonly MqttClientOptions _mqttOptions;

private const string TemperatureTopic = "factory/line1/machine1/temperature";

private const string StatusTopic = "factory/line1/machine1/status";

public MainWindow()

{

InitializeComponent();

var factory = new MqttFactory();

_mqttClient = factory.CreateMqttClient();

_mqttOptions = new MqttClientOptionsBuilder()

.WithTcpServer("localhost", 1883)

.WithClientId("wpf-temperature-dashboard")

.WithCleanSession(false)

.Build();

ConfigureMqttEvents();

}

private void ConfigureMqttEvents()

{

_mqttClient.ConnectedAsync += async e =>

{

await Dispatcher.InvokeAsync(() =>

{

ConnectionStatusText.Text = "Broker Status: Connected";

AddLog("Connected to MQTT broker.");

});

await _mqttClient.SubscribeAsync(new MqttTopicFilterBuilder()

.WithTopic(TemperatureTopic)

.WithQualityOfServiceLevel(MqttQualityOfServiceLevel.AtLeastOnce)

.Build());

await _mqttClient.SubscribeAsync(new MqttTopicFilterBuilder()

.WithTopic(StatusTopic)

.WithQualityOfServiceLevel(MqttQualityOfServiceLevel.AtLeastOnce)

.Build());

await Dispatcher.InvokeAsync(() =>

{

AddLog($"Subscribed to: {TemperatureTopic}");

AddLog($"Subscribed to: {StatusTopic}");

});

};

_mqttClient.DisconnectedAsync += async e =>

{

await Dispatcher.InvokeAsync(() =>

{

ConnectionStatusText.Text = "Broker Status: Disconnected";

AddLog("Disconnected from MQTT broker.");

});

await Task.Delay(TimeSpan.FromSeconds(5));

try

{

await Dispatcher.InvokeAsync(() => AddLog("Trying to reconnect dashboard..."));

await _mqttClient.ConnectAsync(_mqttOptions);

}

catch

{

await Dispatcher.InvokeAsync(() => AddLog("Reconnect failed."));

}

};

_mqttClient.ApplicationMessageReceivedAsync += async e =>

{

var topic = e.ApplicationMessage.Topic;

var payload = Encoding.UTF8.GetString(e.ApplicationMessage.PayloadSegment);

await Dispatcher.InvokeAsync(() => HandleMessage(topic, payload));

};

}

private void HandleMessage(string topic, string payload)

{

if (topic == StatusTopic)

{

MachineStatusText.Text = $"Machine Status: {payload}";

MachineStatusText.Foreground = payload == "ONLINE"

? System.Windows.Media.Brushes.Green

: System.Windows.Media.Brushes.Red;

AddLog($"Status received: {payload}");

return;

}

if (topic == TemperatureTopic)

{

try

{

var reading = JsonSerializer.Deserialize<TemperatureReading>(payload);

if (reading == null) return;

TemperatureText.Text = $"Temperature: {reading.Temperature} °C";

if (reading.Temperature >= 85)

{

AlertText.Text = "Alert: High temperature detected!";

AlertText.Foreground = System.Windows.Media.Brushes.Red;

}

else if (reading.Temperature >= 75)

{

AlertText.Text = "Alert: Warning temperature level.";

AlertText.Foreground = System.Windows.Media.Brushes.Orange;

}

else

{

AlertText.Text = "Alert: Normal";

AlertText.Foreground = System.Windows.Media.Brushes.Green;

}

AddLog($"Temperature received: {reading.Temperature} °C at {reading.TimestampUtc:HH:mm:ss}");

}

catch (Exception ex)

{

AddLog($"Invalid message received: {ex.Message}");

}

}

}

private async void ConnectButton_Click(object sender, RoutedEventArgs e)

{

try

{

if (!_mqttClient.IsConnected)

await _mqttClient.ConnectAsync(_mqttOptions);

}

catch (Exception ex)

{

AddLog($"Connection failed: {ex.Message}");

}

}

private async void DisconnectButton_Click(object sender, RoutedEventArgs e)

{

try

{

if (_mqttClient.IsConnected)

await _mqttClient.DisconnectAsync();

}

catch (Exception ex)

{

AddLog($"Disconnect failed: {ex.Message}");

}

}

private void AddLog(string message)

{

LogListBox.Items.Insert(0, $"{DateTime.Now:HH:mm:ss} - {message}");

}

protected override async void OnClosed(EventArgs e)

{

if (_mqttClient.IsConnected)

await _mqttClient.DisconnectAsync();

base.OnClosed(e);

}

}

public class TemperatureReading

{

public string MachineId { get; set; } = "";

public string LineId { get; set; } = "";

public double Temperature { get; set; }

public DateTime TimestampUtc { get; set; }

}

# 9. How to Run Both Projects in Parallel

## 9.1 Start Broker First

cd "C:\Program Files\mosquitto"

mosquitto.exe -c mosquitto.conf -v

## 9.2 Configure Multiple Startup Projects in Visual Studio

Right-click the solution MqttTemperatureDemo.

Select Properties.

Go to Common Properties > Startup Project.

Choose Multiple startup projects.

Set TemperatureDashboard to Start.

Set TemperatureSensorSimulator to Start.

Click Apply and OK.

Press F5.

Important runtime order Broker must run first. Then WPF and sensor can run in parallel. The sensor does not send directly to WPF. Both communicate through the MQTT broker.

# 10. Manual Broker Test

You can test Mosquitto before running .NET apps.

Terminal 1 - subscribe:

cd "C:\Program Files\mosquitto"

mosquitto_sub.exe -h localhost -t factory/line1/machine1/temperature -v

Terminal 2 - publish:

cd "C:\Program Files\mosquitto"

mosquitto_pub.exe -h localhost -t factory/line1/machine1/temperature -m "85"

If Terminal 1 shows the value 85, your broker is working.

# 11. When to Use MQTT

IoT devices and sensors

Manufacturing telemetry

Smart meters

GPS and vehicle tracking

Low-bandwidth networks

Battery-powered devices

Event-driven device-to-cloud communication

Unstable network environments

# 12. When Not to Use MQTT

Large file transfer such as images, video, or backups. Use object storage or file transfer protocols instead.

Business request-response APIs such as CreateOrder or GetCustomer. Use REST or gRPC instead.

High-volume event streaming with replay, partitions, and long retention. Use Kafka or a similar streaming platform.

Complex querying or analytics directly on messages. Store data in a database or data lake first.

Strict global ordering across many topics. MQTT ordering is generally per client/topic flow, not a global ordered log.

# 13. Production Issues and How to Solve Them

| Production Issue | Cause | Solution |
| --- | --- | --- |
| Duplicate messages | QoS 1 can redeliver messages if acknowledgement is lost. | Make processing idempotent. Add messageId, timestamp, and deduplication cache. |
| Broker CPU high | Too many wildcard subscriptions such as factory/# can cause excessive fan-out. | Design topic hierarchy carefully. Split by plant, line, machine, or tenant. Scale broker cluster. |
| Offline device flood | Device reconnects and sends thousands of buffered messages. | Use rate limiting, batching, backpressure, and max queue limits. |
| Lost connectivity | Factory network or device network is unstable. | Use keep-alive, auto reconnect, persistent sessions, and Last Will messages. |
| Security risk | Anonymous plain TCP MQTT is unsafe for production. | Use TLS, username/password, device certificates, ACLs, and private networking. |

# 14. Trade-Offs

| Advantages | Limitations |
| --- | --- |
| Very lightweight Low bandwidth Good for sensor telemetry Decouples publishers and subscribers Supports QoS levels Supports retained messages and LWT Good for unreliable networks | Broker can become bottleneck QoS 1 can produce duplicates QoS 2 adds latency No Kafka-style replay by default Not suitable for large files Topic design and ACL design are critical |

# 15. Senior Developer Interview Answer

30-second answer MQTT is a lightweight publish-subscribe protocol mainly used for IoT and telemetry. Devices publish messages to topics and consumers subscribe through a broker. In a manufacturing use case, sensors can publish temperature every second to topics like factory/line1/machine1/temperature, while dashboards or processing services consume those messages. I would use QoS 1 for important telemetry, retained messages for latest machine status, and Last Will to detect unexpected device disconnection. MQTT is excellent for low-bandwidth device communication, but I would not use it for large file transfer, business CRUD APIs, or Kafka-style event replay workloads.

Production story example In one factory telemetry scenario, frequent wildcard subscriptions such as factory/# caused high broker CPU because every message was fan-out to many consumers. We redesigned topics into line-specific and service-specific subscriptions, reduced wildcard usage, added consumer filtering, and scaled brokers horizontally. This reduced broker CPU and improved dashboard latency.

# 16. Quick Troubleshooting Checklist

Check Mosquitto is running on localhost:1883.

If connection fails, verify mosquitto.conf has listener 1883 and allow_anonymous true for local learning.

Check Windows Firewall if port 1883 is blocked.

Run netstat -ano | findstr 1883 to confirm port listening.

Ensure both projects use the same MQTTnet version.

Make sure WPF updates UI using Dispatcher from MQTT callback.

Use different ClientId values for sensor and dashboard.

Do not use the same ClientId for two running clients because broker may disconnect the older one.